Deployment Document

Project

OCP 4.15 DR Cluster Deployment

v1.1

Client: Bank-ABC

Integra Micro Systems Pvt. Ltd.

No. 4, 1st Floor, Bellary Road,

12th KM, Jakkur,

Bangalore 560 064, INDIA

Tel: 080-46632400

www.integramicro.com

Disclaimer

Copyright © 2013:  Integra Micro Systems Private Limited, Registered Office #4, Bellary Rd, 12, KM, Yashoda Nagar, Jakkur, Bangalore, Karnataka India.

All rights about this document are reserved and  shall not be , in whole or in part, copied, photocopied, reproduced, translated, or reduced to any manner including but not limited to electronic, mechanical, machine readable ,photographic, optic recording or otherwise without prior consent, in writing, of Integra Micro Systems Private Limited (the Company).

The information in this document is subject to changes without notice. This describes only the product defined in the introduction of this documentation. This document is intended for the use of prospective customers of the Company Products Solutions and or Services for the sole purpose of the transaction for which the document is submitted. No part of it may be reproduced or transmitted in any form or manner whatsoever without the prior written permission of the company. The Customer, who/which assumes full responsibility for using the document appropriately. The Company welcomes customer comments as part of the process of continuous development and improvement.

The Company, has made all reasonable efforts to ensure that the information contained in the document are adequate, sufficient and free of material errors and omissions. The Company will, if necessary, explain issues, which may not be covered by the document. However, the Company does not assume any liability of whatsoever nature, for any errors in the document except the responsibility to provide correct information when any such error is brought to company’s knowledge. The Company will not be responsible, in any event, for errors in this document or for any damages, incidental or consequential, including monetary losses that might arise from the use of this document or of the information contained in it.

This document and the Products, Solutions and Services it describes are intellectual property of the Company and/or of the respective owners thereof, whether such IPR is registered, pending for registration, applied for registration or not.

The only warranties for the Company Products, Solutions and Services are set forth in the express warranty statements accompanying its products and services. Nothing here in should be construed as constituting an additional warranty. The Company shall not be liable for technical or editorial errors or omissions contained herein.

The Company logo is a trademark of the Company. Other products, names, logos mentioned in this document, if any, may be trademarks of their respective owners.

Copyright © 2013:  Integra Micro Systems Private Limited. All rights reserved.

Release History

| - Release Version | - Release Date | - Description of the release | - Created By | - Approved By |
| --- | --- | --- | --- | --- |
| 0.1 | - 10-09-2024 | - First Draft | - Vageesh/Sowjanya/Kiran | - Murali.K.Muddada |
| 0.2 | 12-09-2024 | Updated ELK details | - Vageesh/Sowjanya/Kiran | - Murali.K.Muddada |
| 0.3 | 13-09-2024 | Updated prometheous and Graphana details | - Vageesh/Sowjanya/Kiran | - Murali.K.Muddada |
| 0.4 | 16-09-2024 | Added Cronjob for ETCD backup | - Vageesh/Sowjanya/Kiran | - Murali.K.Muddada |

- Confidentiality Statement

Copyright © 2011-2012, Integra Micro Systems Private Limited. All rights reserved. This product or document may not, in whole or in part, be copied, photocopied, reproduced, translated, or reduced to any electronic medium or machine readable form, by any means electronic, mechanical, photographic, optic recording or otherwise without prior consent, in writing, of the copyright owner. Statutory declaration under section 52A of the Copyright Act 1957.

Table of Contents

1  About the Guide	6

1.1 Target Audience	6

1.2 Acronyms and Abbreviations	6

2 Introduction	8

2.1 Topics Covered	8

2.2 Scope	8

2.3 Out of Scope	8

3 Implementation Details	9

3.1 Environment Setup	9

3.1.1 Network Information	9

3.1.2 Network Services	9

3.1.3 Node Information	9

3.1.4 Storage Information	9

3.1.5 Certificates	9

4 Technical Implementation	10

4.1 Check Bastion Node for all the pre-Installation requirements	10

4.1.1 Operating System Details	10

4.1.2 Block Device (Hard Disk) Details	10

4.1.3 Network Configuration	10

4.1.4 Route and Default Gateway details	10

4.1.5 Enabling Repositories	10

4.2 Installing a Connected Cluster	11

4.2.1 Configure Environmental Services in Bastion.	11

4.2.2 Generating an SSH private key and adding it to the agent	13

4.2.3 Create install-config.yaml file	13

4.2.4 Creating the Openshift/Kubernetes manifest	15

4.2.5 Create the Ignition config files	15

4.3 Deploying Openshift	16

4.3.1 Monitoring the Bootstrap Process	16

4.3.2 Remove the Bootstrap Node	16

4.3.3 Join Nodes	16

4.4 Accessing the Cluster	17

4.4.1 Accessing the Openshift Console	17

4.4.2 Accessing the Cluster Nodes.	17

4.5 Deploying Openshift Internal Registry with Persistent Storage	19

4.6 Configure System Clock Sync with NTP Server	19

4.6.1 Configure master nodes system clock sync with NTP server	20

4.7 Configuring an HTPasswd identity provider	21

4.8 Registering the Cluster	22

4.9 Encrypting the ETCD data	24

4.10 Backing Up ETCD data	24

4.11 Deploying and Configuring Openshift Logging	25

4.11.1 Install Cluster Logging Operators	25

4.11.2 Create Cluster Logging Instance	26

4.11.3 Viewing cluster logs from the Kibana dashboard	28

4.12 Applying a custom Alertmanager configuration with the SMTP details	29

4.13 Removing the kubeadmin user	30

4.14 Moving monitoring components from Worker to Infra labeled nodes	31

4.15 Configuring the default scheduler to control pod placement	33

4.16 Etcd backup Cronjob	33

# About the Guide

This design document currently includes a Prod environment which will be hosted on Virtual Machines.

## Target Audience

This document is intended for Client technical staff responsible for the environment.

## Acronyms and Abbreviations

The table below provides a glossary of the terms and acronyms used within this document.

| Acronym | Description |
| --- | --- |
| RH | Red Hat, Inc |
| OKE | Openshift Kubernetes Engine |
| OCP | Openshift Container Platform |
| AD | Active Directory |
| API | Application Programming Interface |
| AZ | Availability Zone |
| CA | Certificate Authority |
| CR | Custom Resource |
| DC | Data Centre |
| DNS | Domain Name System |
| DHCP | Dynamic Host Configuration Protocol |
| DVR | Distributed Virtual Routers, a networking configuration for routing using Compute and Network nodes for separate networking functions |
| FQDN | Fully Qualified Domain Name |
| Guest | Also see “VM”. This is virtual machine running on a Host. |
| HA | High-Availability or Highly-Available |
| Host | The physical hardware or the logical OS which runs virtualisation technology allowing one or more Guest OS’s to run on the hardware owned by the Host |
| KVM | Kernel-based Virtual Machine |
| L2 | Layer 2, part of the TCP/IP Network Stack |
| L3 | Layer 3, part of the TCP/IP Network Stack |
| LBaaS | Load Balancing as a Service |
| NAT | Network Address Translation |
| NIC | Network Interface Card. References a virtual or a physical port allowing network access and interface to a Host or Guest VM. |
| NTP | Network Time Protocol |
| OCP | OpenShift Container Platform |
| OS | Operating System |
| OVS | Open vSwitch, a Linux-based virtual switch appliance used in OpenStack |
| PXE | Preboot Execution Environment |
| QA | Quality Assurance |
| SAML/SAML2 | A protocol allowing authentication and federation of multiple domains |
| SAN | Storage Area Network |
| SSL | Secure Sockets Layer |
| TLS | Transport Layer Security |
| VIP | Virtual IP address |
| VLAN | Virtual LAN is a networking virtualisation technology |
| VM | Virtual machine, in OSP terms, synonymous with “Workload” or “Guest” |
| VXLAN | Virtual Extensible LAN (VXLAN) is a network virtualisation technology |
| Workload | Synonym for “Guest” or “VM” |
| K8s | Kubernetes |
| RHCOS | Red Hat CoreOS |
| DIY | Do It Yourself |
| VM | Virtual Machine |
| OCR | OpenShift Container Registry (Integrated Registry) |

# Introduction

## Topics Covered

Openshift topology - Discussion about OCP high level architecture, subsystems as well as high-level overview of process, technology and operations.

Design guidelines and architecture - Interactive discussion covering minimal HA architectures for development environment and constraints.

## Scope

The scope of this document is to describe the architecture that Red Hat can offer for Openshift. This document is the result of the design workshop and should cover all requirements that Bank ABC put forward during the workshop. As a part of this project engagement with Bank ABC, Red Hat team will:

●	Design and validate Openshift Kubernetes Engine (OKE) 4.15 in prod Datacenters.

## Out of Scope

This document limits the discussions and recommendations only to Red Hat Components involved for Bank ABC.

# Implementation Details

## Environment Setup

Steps to prepare or create the environment for the proposed architecture

### Network Information

Table 1. Network Data

| Name | IP Address Pool | Comments |
| --- | --- | --- |
| Cluster Network | 10.128.0.0/14 | Non Routable |
| Service Network | 172.30.0.0/16 | Non Routable |

### Network Services

Table 2. Network Service Data

| Services | Host | Comments |
| --- | --- | --- |
| DNS Server | 10.151.20.201 |  |
| DNS Server | 10.151.20.202 |  |

### Node Information

Table 3. Nodes Data

| Server FQDN | IP | Role | Subs Used | OS |
| --- | --- | --- | --- | --- |
| drlonvsacmdrcm1.acmdrc.arabbanking.local | 10.151.24.78 | Master/Control-Plane | N/A | RHCOS |
| drlonvsacmdrcm2.acmdrc.arabbanking.local | 10.151.24.79 | Master/Control-Plane | N/A | RHCOS |
| drlonvsacmdrcm3.acmdrc.arabbanking.local | 10.151.24.81 | Master/Control-Plane | N/A | RHCOS |
| drlonvsacmdrcw1.acmdrc.arabbanking.local | 10.151.24.82 | Worker | N/A | RHCOS |
| drlonvsacmdrcw2.acmdrc.arabbanking.local | 10.151.24.83 | Worker | N/A | RHCOS |
| drlonvsacmdrcw3.acmdrc.arabbanking.local | 10.151.24.84 | Infra | N/A | RHCOS |
| drlonvsacmdrcw4.acmdrc.arabbanking.local | 10.151.24.85 | Infra | N/A | RHCOS |

### Storage Information

Table 4. Storage Data

| Application | Storage Type | Storage Details | Size |
| --- | --- | --- | --- |
| Registry | NFS | NFS_Volume | 500GB |
| ELK Application Deployment | NFS | NFS_Volume | 3*300GB |
| Prometheous and Graphana | NFS | NFS_Volume | 2*300GB, 2*5GB |

### Certificates

Bank ABC has planned to use certificates signed by their internal CA for Wild Card (*.apps) Domain Ingress Controller.

# Technical Implementation

## Check Bastion Node for all the pre-Installation requirements

### Operating System Details

• Bastion Node

[core@drlonvsacmdrcmg ]$ cat /etc/redhat-release Red Hat Enterprise Linux release 8.10(Ootpa)

### Block Device (Hard Disk) Details

• Bastion Node

[core@drlonvsacmdrcmg ~]$ df -hT Filesystem Type Size Used Avail Use% Mounted on devtmpfs devtmpfs 7.7G 0 7.7G 0% /dev tmpfs tmpfs 7.7G 252K 7.7G 1% /dev/shm tmpfs tmpfs 7.7G 1.1M 7.7G 1% /run tmpfs tmpfs 7.7G 0 7.7G 0% /sys/fs/cgroup /dev/mapper/rhel_rhel8-root xfs 15G 5.6G 9.5G 38% / /dev/mapper/rhel_rhel8-tmp xfs 10G 104M 9.9G 2% /tmp /dev/mapper/rhel_rhel8-var xfs 10G 1.2G 8.9G 12% /var /dev/mapper/rhel_rhel8-var_log xfs 10G 1.4G 8.7G 14% /var/log /dev/mapper/rhel_rhel8-var_tmp xfs 10G 104M 9.9G 2% /var/tmp /dev/mapper/software--vg-software_vol xfs 100G 1.9G 99G 2% /software /dev/sda1 xfs 1014M 360M 655M 36% /boot /dev/mapper/rhel_rhel8-var_log_audit xfs 15G 176M 15G 2% /var/log/audit /dev/mapper/rhel_rhel8-home xfs 10G 8.2G 1.9G 82% /home tmpfs tmpfs 1.6G 24K 1.6G 1% /run/user/4029 tmpfs tmpfs 1.6G 48K 1.6G 1% /run/user/4030 10.151.24.41:/ACM_OC_DRC nfs4 100G 289M 100G 1% /acmdata tmpfs tmpfs 1.6G 0 1.6G 0% /run/user/4031 tmpfs tmpfs 1.6G 0 1.6G 0% /run/user/0 tmpfs tmpfs 1.6G 0 1.6G 0% /run/user/4028

### Network Configuration

- Bastion Node

[core@drlonvsacmdrcmg ~]$ cat /etc/resolv.conf # Generated by NetworkManager search arabbanking.local nameserver 10.151.20.201 nameserver 10.151.20.202

### Route and Default Gateway details

- Bastion Node

[core@drlonvsacmdrcmg ~]$ route -n Kernel IP routing table Destination Gateway Genmask Flags Metric Ref Use Iface 0.0.0.0 10.151.27.254 0.0.0.0 UG 100 0 0 ens192 10.151.24.0 0.0.0.0 255.255.252.0 U 100 0 0 ens192

### Enabling Repositories

Bank ABC Team created a VM with RHEL 8.10 with minimal install which is registered with their internal Red Hat Satellite Server and the following repositories has been enabled by them.

• Bastion Node

[core@ drlonvsacmdrcmg ~]$ yum repolist Not root, Subscription Management repositories not updated repo id repo name rhel-8-for-x86_64-appstream-rpms Red Hat Enterprise Linux 8 for x86_64 - AppStream (RPMs) rhel-8-for-x86_64-baseos-rpms Red Hat Enterprise Linux 8 for x86_64 - BaseOS (RPMs)

## Installing a Connected Cluster

### Configure Environmental Services in Bastion.

1. Install & configure HAProxy

dnf install haproxy -y

[root@drlonvsacmlb1 ~]$ cat /etc/haproxy/haproxy.cfg

# Global settings #--------------------------------------------------------------------- global maxconn 20000 log /dev/log local0 info chroot /var/lib/haproxy pidfile /var/run/haproxy.pid user haproxy group haproxy daemon # turn on stats unix socket stats socket /var/lib/haproxy/stats #--------------------------------------------------------------------- # common defaults that all the 'listen' and 'backend' sections will # use if not designated in their block #--------------------------------------------------------------------- defaults log global mode http option httplog option dontlognull option http-server-close option redispatch option forwardfor except 127.0.0.0/8 retries 3 maxconn 20000 timeout http-request 10000ms timeout http-keep-alive 10000ms timeout check 10000ms timeout connect 40000ms timeout client 300000ms timeout server 300000ms timeout queue 50000ms # Enable HAProxy stats listen stats bind :9000 stats uri /stats stats refresh 10000ms # Kube API Server frontend k8s_api_frontend bind :6443 default_backend k8s_api_backend mode tcp backend k8s_api_backend mode tcp balance source # server drlonvsacmbst.acmdrc.arabbanking.local 10.151.24.91:6443 check server drlonvsacmdrcm1.acmdrc.arabbanking.local 10.151.24.78:6443 check server drlonvsacmdrcm2.acmdrc.arabbanking.local 10.151.24.79:6443 check server drlonvsacmdrcm3.acmdrc.arabbanking.local 10.151.24.81:6443 check # OCP Machine Config Server frontend ocp_machine_config_server_frontend mode tcp bind :22623 default_backend ocp_machine_config_server_backend backend ocp_machine_config_server_backend mode tcp balance source # server drlonvsacmbst.acmdrc.arabbanking.local 10.151.24.91:22623 check server drlonvsacmdrcm1.acmdrc.arabbanking.local 10.151.24.78:22623 check server drlonvsacmdrcm2.acmdrc.arabbanking.local 10.151.24.79:22623 check server drlonvsacmdrcm3.acmdrc.arabbanking.local 10.151.24.81:22623 check # OCP Ingress - layer 4 tcp mode for each. Ingress Controller will handle layer 7. frontend ocp_http_ingress_frontend bind :80 default_backend ocp_http_ingress_backend mode tcp backend ocp_http_ingress_backend balance source mode tcp server drlonvsacmdrcw1.acmdrc.arabbanking.local 10.151.24.82:80 check server drlonvsacmdrcw2.acmdrc.arabbanking.local 10.151.24.83:80 check frontend ocp_https_ingress_frontend bind *:443 default_backend ocp_https_ingress_backend mode tcp backend ocp_https_ingress_backend mode tcp balance source server drlonvsacmdrcw1.acmdrc.arabbanking.local 10.151.24.82:443 check server drlonvsacmdrcw2.acmdrc.arabbanking.local 10.151.24.83:443 check

Apply HAProxy configuration

2. Enable and start the service

setsebool -P haproxy_connect_any 1 # SELinux name_bind access systemctl enable haproxy systemctl start haproxy systemctl status haproxy

- Install & configure Apache Web Server

dnf install httpd -y

- Change default listen port to 8080 in httpd.conf

sed -i 's/Listen 80/Listen 0.0.0.0:8080/' /etc/httpd/conf/httpd.conf

- Enable and start the service

systemctl enable httpd systemctl start httpd systemctl status httpd

- Making a GET request to localhost on port 8080 should now return the default Apache webpage

curl localhost:8080

### Generating an SSH private key and adding it to the agent

- Generate SSH Key

- Start the ssh-agent process as a background task

[core@drlonvsacmdrcmg~]$ ssh-keygen [core@drlonvsacmdrcmg~]$eval "$(ssh-agent -s)"

- Add your SSH private key to the ssh-agent

[[core@drlonvsacmdrcmg ~]$ ssh-add /home/core/.ssh/id_rsa Identity added: /home/core/.ssh/id_rsa (/home/core/.ssh/id_rsa)

### Create install-config.yaml file

[core@drlonvsacmdrcmg ~]$ cat install-config.yaml apiVersion: v1 baseDomain: arabbanking.local compute: - hyperthreading: Enabled name: worker replicas: 0 controlPlane: hyperthreading: Enabled name: master replicas: 3 metadata: name: acmdrc networking: clusterNetwork: - cidr: 10.128.0.0/14 hostPrefix: 23 networkType: OVNKubernetes serviceNetwork: - 172.30.0.0/16 platform: none: {} fips: false pullSecret: '{"auths":{"cloud.openshift.com":{"auth":"b3BlbnNoaWZ0LXJlbGVhc2UtZGV2K29jbV9hY2Nlc3NfN2YwMjY5YmMzMTVkNGQ0MGEyZWVlMWY0MjRiYjFiMjc6TU5XRERFWjk2WDlDOUE5TEg5TEZHTlNJT0lFMkwxQko5QkxOTE5XOU9KVFExQjJLSVVGRUM4TUNTT01TUDlSMg==","email":"prem.ram@bank-abc.com"},"quay.io":{"auth":"b3BlbnNoaWZ0LXJlbGVhc2UtZGV2K29jbV9hY2Nlc3NfN2YwMjY5YmMzMTVkNGQ0MGEyZWVlMWY0MjRiYjFiMjc6TU5XRERFWjk2WDlDOUE5TEg5TEZHTlNJT0lFMkwxQko5QkxOTE5XOU9KVFExQjJLSVVGRUM4TUNTT01TUDlSMg==","email":"prem.ram@bank-abc.com"},"registry.connect.redhat.com":{"auth":"fHVoYy0xZ295RE1HeUFGTVlKbUdzNUNuZDVBZ2ZRS2o6ZXlKaGJHY2lPaUpTVXpVeE1pSjkuZXlKemRXSWlPaUl4WVRReFpHRm1ZemczTXprME9HSmlZalk1TWpsaU5UWmxaVGN5TlRsaE1DSjkuUUJsekt5MjlQeThnUHVpZkVtVnVUV0pfREx4WFBzbWZwYTFPN1lJN1NyU2lvZk1lZ2ZfSEx2d0NaLXd4MmhNQnp3WExHdlFLTUc4XzJqTnZTNEQ2OUxzUk1UNi1fbXFEcVJSdWNXb3VLV3lBYkNWX2dkVnBiVVQyemJ0bVExbWxEZXVCRE10LTdlaHh5eTdSdXVFeUhmemdLRHdNYVBmTnlBdTJ0cTQ0a2x4NExVY3l0MGVsZ2ZubUpOUUZXaGhPZHRKU01yVmlNS3NvYjdwdm5iT0xBbURMZ0hFZnp4Sy1EUHJlcUpwLUtrQXhNZ3UtTFdvQUV1czRkNjBTbi1fSTVrcHlNU1E2ZTZmbEhWWW9UQ0R1dFZvZEltZ0stczhGcGRudWk1TzJOaS1TdkJ6TFNtTloxSkZtSG5XeWRNdVQ0UzZQcTh6cm9GWEZTVDZpc0QyZVFmVVN6endOT2VYSWNzQXF0ZC1iY0U1bm04Qk1IY2Yxd3BENTRmb1BfOFFad2JCQzRHVzBNZzU4RktsYTRDQkxab2VGVk5Bd2hpdzF4SUFCcjBLdGF5cVlla0pvbjVHbVF2a2dvRUhucmJPRDFlM0xLMmZCbVR4b19valVtRkNNT09NSzU3S1E2NGQwaXRrNUJRM0NxYURIY053VUF3VnZrWFZXNExTdHh4T2QzV1UzS1lOQlNmX1dFWThKT0RySjB6ZTFVWTcxV1VLeXdzMHJWMXcyeXlmci1XMGQtRE42dWxvMzJ1T01WbnRwcXZQWG5qTUZySkVRdmJoaHY1TktKQkpOVXhPRUUxdU9nUUpBTE11RUR6eWpFSG0xbzROZTloTXdrSnpoR2RWcTg4QVRlQ203dzJkWEJpc0hLanY4cWpjdWgweXlpSTRjWDhkT1NJSUE5eVU=","email":"prem.ram@bank-abc.com"},"registry.redhat.io":{"auth":"fHVoYy0xZ295RE1HeUFGTVlKbUdzNUNuZDVBZ2ZRS2o6ZXlKaGJHY2lPaUpTVXpVeE1pSjkuZXlKemRXSWlPaUl4WVRReFpHRm1ZemczTXprME9HSmlZalk1TWpsaU5UWmxaVGN5TlRsaE1DSjkuUUJsekt5MjlQeThnUHVpZkVtVnVUV0pfREx4WFBzbWZwYTFPN1lJN1NyU2lvZk1lZ2ZfSEx2d0NaLXd4MmhNQnp3WExHdlFLTUc4XzJqTnZTNEQ2OUxzUk1UNi1fbXFEcVJSdWNXb3VLV3lBYkNWX2dkVnBiVVQyemJ0bVExbWxEZXVCRE10LTdlaHh5eTdSdXVFeUhmemdLRHdNYVBmTnlBdTJ0cTQ0a2x4NExVY3l0MGVsZ2ZubUpOUUZXaGhPZHRKU01yVmlNS3NvYjdwdm5iT0xBbURMZ0hFZnp4Sy1EUHJlcUpwLUtrQXhNZ3UtTFdvQUV1czRkNjBTbi1fSTVrcHlNU1E2ZTZmbEhWWW9UQ0R1dFZvZEltZ0stczhGcGRudWk1TzJOaS1TdkJ6TFNtTloxSkZtSG5XeWRNdVQ0UzZQcTh6cm9GWEZTVDZpc0QyZVFmVVN6endOT2VYSWNzQXF0ZC1iY0U1bm04Qk1IY2Yxd3BENTRmb1BfOFFad2JCQzRHVzBNZzU4RktsYTRDQkxab2VGVk5Bd2hpdzF4SUFCcjBLdGF5cVlla0pvbjVHbVF2a2dvRUhucmJPRDFlM0xLMmZCbVR4b19valVtRkNNT09NSzU3S1E2NGQwaXRrNUJRM0NxYURIY053VUF3VnZrWFZXNExTdHh4T2QzV1UzS1lOQlNmX1dFWThKT0RySjB6ZTFVWTcxV1VLeXdzMHJWMXcyeXlmci1XMGQtRE42dWxvMzJ1T01WbnRwcXZQWG5qTUZySkVRdmJoaHY1TktKQkpOVXhPRUUxdU9nUUpBTE11RUR6eWpFSG0xbzROZTloTXdrSnpoR2RWcTg4QVRlQ203dzJkWEJpc0hLanY4cWpjdWgweXlpSTRjWDhkT1NJSUE5eVU=","email":"prem.ram@bank-abc.com"}}}' sshKey: 'ssh ed25519AAAAC3NzaC1lZDI1NTE5AAAAINiuKrt0X3C3+3sreFbLXZ9kRUUEnZWS8VPQwGpyKG6T core@drlonvsacmdrcmg'

### Creating the Openshift/Kubernetes manifest

[core@drlonvsacmdrcmg]$ openshift-install create manifests INFO Consuming Install Config from target directory WARNING Mak ing control-plane schedulable by setting MastersSchedulable to true for Scheduler cluster settings INFO Manifests created in: /home/core/clusterconfig/manifests and /home/core/clusterconfig/openshift

### Create the Ignition config files

[core@drlonvsacmdrcmg]$ openshift-install create ignition-configs INFO Consuming Openshift Manifests from target directory INFO Consuming Master Machines from target directory INFO Consuming OpenShift Install (Manifests) from target directory INFO Consuming Worker Machines from target directory INFO Consuming Common Manifests from target directory INFO Ignition-Configs created in: . and auth

The following files will be generated in the directory.

[core@drlonvsacmdrcmg]$ tree . ├── auth │ ├── kubeadmin-password │ └── kubeconfig ├── bootstrap.ign ├── master.ign ├── metadata.json └── worker.ign

• Install HTTPD Package • Start and Enable httpd Service

[core@drlonvsacmdrcmg~]# yum install httpd –y [core@drlonvsacmdrcmg~]# systemctl start httpd [core@drlonvsacmdrcmg~]# systemctl enable httpd [core@drlonvsacmdrcmg~]# systemctl status httpd ● httpd.service - The Apache HTTP Server Loaded: loaded (/usr/lib/systemd/system/httpd.service; enabled; vendor preset: disabled) Active: active (running) Docs: man:httpd(8) man:apachectl(8) Main PID: 25568 (httpd) • Verify the firewalld.service status [core@bastion ~]# systemctl status firewalld ● firewalld.service - firewalld - dynamic firewall daemon Loaded: loaded (/usr/lib/systemd/system/firewalld.service; disabled; vendor preset: enabled) Active: inactive (dead) Docs: man:firewalld(1)

• Create a directory in /var/www/html/ and copy the ignition files

[core@drlonvsacmdrcmg~]# mkdir -p /var/www/html/ocp4/

• Copy bootstrap.ign from /home/clusterconfig/ to /var/www/html/ocp4/
directory

[core@drlonvsacmdrcmg~]# cp /home/clusterconfig//bootstrap.ign /var/www/html/ocp4/

• Make sure bootstrap.ign file has 644 permissions

[core@drlonvsacmdrcmg~]# chmod 644 /var/www/html/ocp4/bootstrap.ign

## Deploying Openshift

Provisioning bootstrap node

sudo coreos-installer install /dev/sda --insecure-url http://drlonvsacmdrcmg.acmdrc.arabbanking.local:8080/ocp/bootstrap.ign --insecure-ignition --copy-network

Run the above command to provision the master/worker node with respect to their ignition files.

### Monitoring the Bootstrap Process

Monitoring the bootstrap process from the Bastion host at different log levels (debug, error, info)

~/openshift-install --dir ~/ocp-install wait-for bootstrap-complete --log-level=debug

### Remove the Bootstrap Node

Wait for installation to complete

- Collect the OpenShift Console address and kubeadmin credentials from the output of the install-complete event

[dbpuser@ drlonvsacmdrcmg ~]$ ./openshift-install --dir ~/clusterconfig wait-for install-complete INFO Waiting up to 40m0s (until 11:23AM) for the cluster at https://api.acmdrc.arabbanking.local:6443 to initialize... INFO Checking to see if there is a route at openshift-console/console... INFO Install complete! INFO To access the cluster as the system:admin user when using 'oc', run 'export KUBECONFIG=/home/dbpuser/clusterconfig/auth/kubeconfig' INFO Access the OpenShift web-console here: https://console-openshift-console.apps.acmprod.arabbanking INFO Login to the console with user: "kubeadmin", and password: "******" INFO Time elapsed: 1s

- Continue to join the mater or worker nodes to the cluster in a new tab whilst waiting for the above command to complete.

### Join Nodes

- Setup 'oc' and 'kubectl' clients on the bastion machine.

export KUBECONFIG=~/ocp-install/auth/kubeconfig # Test auth by viewing cluster nodes oc get nodes

- View and approve pending CSRs.

Note: Once you approve the first set of CSRs additional 'kubelet-serving' CSRs will be created. These must be approved too. If you do not see pending requests wait until you do.

# View CSRs oc get csr # Approve all pending CSRs oc get csr -o go-template='{{range .items}}{{if not .status}}{{.metadata.name}}{{"\n"}}{{end}}{{end}}' ¦ xargs oc adm certificate approve # Wait for kubelet-serving CSRs and approve them too with the same command oc get csr -o go-template='{{range .items}}{{if not .status}}{{.metadata.name}}{{"\n"}}{{end}}{{end}}' ¦ xargs oc adm certificate approve

- Watch and wait for the Nodes to join the cluster and enter a 'Ready' status.

watch oc get nodes

[core@drlonvsacmdrcmg ~]$ oc get nodes NAME STATUS ROLES AGE VERSION drlonvsacmdrcm1.acmdrc.arabbanking.local Ready control-plane,master 7d v1.28.11+add48d0 drlonvsacmdrcm2.acmdrc.arabbanking.local Ready control-plane,master 7d v1.28.11+add48d0 drlonvsacmdrcm3.acmdrc.arabbanking.local Ready control-plane,master 7d v1.28.11+add48d0 drlonvsacmdrcw1.acmdrc.arabbanking.local Ready worker 6d21h v1.28.11+add48d0 drlonvsacmdrcw2.acmdrc.arabbanking.local Ready worker 6d21h v1.28.11+add48d0 drlonvsacmdrcw3.acmdrc.arabbanking.local Ready infra 6d21h v1.28.11+add48d0 drlonvsacmdrcw4.acmdrc.arabbanking.local Ready infra 6d3h v1.28.11+add48d0

## Accessing the Cluster

### Accessing the Openshift Console

- Wait for the 'console' Cluster Operator to become available

oc get co

- Navigate to the OpenShift Console URL and log in as the 'admin' user.

You will get self-signed certificate warnings that you can ignore if you need to login as kubeadmin and need to the password again you can retrieve it with: cat ~/clusterconfig/auth/kubeadmin-password.

### Accessing the Cluster Nodes.

- Setup 'oc' and 'kubectl' clients on the bastion machine.

export KUBECONFIG=~/clusterconfig/auth/kubeconfig # Test auth by viewing cluster nodes oc get nodes

- SSH to Master / Worker Node by the following command:

ssh core@<hostname or ip>

- Verify the status of nodes.

[core@drlonvsacmdrcmg ~]$ oc get nodes NAME STATUS ROLES AGE VERSION drlonvsacmdrcm1.acmdrc.arabbanking.local Ready control-plane,master 7d v1.28.11+add48d0 drlonvsacmdrcm2.acmdrc.arabbanking.local Ready control-plane,master 7d v1.28.11+add48d0 drlonvsacmdrcm3.acmdrc.arabbanking.local Ready control-plane,master 7d v1.28.11+add48d0 drlonvsacmdrcw1.acmdrc.arabbanking.local Ready worker 6d21h v1.28.11+add48d0 drlonvsacmdrcw2.acmdrc.arabbanking.local Ready worker 6d21h v1.28.11+add48d0 drlonvsacmdrcw3.acmdrc.arabbanking.local Ready infra 6d21h v1.28.11+add48d0 drlonvsacmdrcw4.acmdrc.arabbanking.local Ready infra 6d3h v1.28.11+add48d0

- Check the status of cluster operators

## Deploying Openshift Internal Registry with Persistent Storage

- By default, there will be no registry pod running as the registry operator’s "managementState" will be set as "Removed". Configure image registry to "Managed" state and move the registry pods to the dedicated infra nodes as shown below. Right now an object storage volume is being used inside the registry pods.

oc edit configs.imageregistry.operator.openshift.io/cluster

- Verify if the registry pod is running on one of the dedicated infra aux nodes as shown below.

[core@drlonvsacmdrcmg ~ ~]$ oc get pods -o wide ¦ grep "image-registry-88b86cdb4-mzbqk" image-registry-88b86cdb4-mzbqk 1/1 Running 1 6d15h 10.139.0.4 drlonvsacmdrcm1.acmdrc.arabbanking.local <none> <none> [dbpuser@bastion ~]$

## Configure System Clock Sync with NTP Server

### Configure master nodes system clock sync with NTP server

- Create a Butane config including the contents of the chrony.conf file, to configure chrony on worker nodes, create a 99-worker-chrony.bu file and use Butane to generate a MachineConfig object file, 99- worker-chrony.yaml, containing the configuration to be delivered to the nodes. Repeat the same steps for other machine-config-pool like master. .

[core@drlonvsacmdrcmg NTP]$ cat worker-chrony.bu variant: openshift version: 4.15.0 metadata: name: 99-worker-chrony labels: machineconfiguration.openshift.io/role: worker storage: files: - path: /etc/chrony.conf mode: 0644 overwrite: true contents: inline: ¦ server 10.151.217.1 iburst driftfile /var/lib/chrony/drift makestep 1.0 3 rtcsync logdir /var/log/chrony

[core@drlonvsacmdrcmg NTP]$ ../butane 99-worker-chrony.bu -o 99-worker-chrony.yaml

- Apply the MachineConfig YAML file

[core@drlonvsacmdrcmg NTP]$oc create -f ./99-worker-chrony.yaml machineconfig.machineconfiguration.openshift.io/99-worker-chrony created [core@drlonvsacmdrcmg NTP]$ pwd /home/T910013790/NTP [core@drlonvsacmdrcmg NTP]$ [core@drlonvsacmdrcmg NTP]$ [core@drlonvsacmdrcmg NTP]$ [core@drlonvsacmdrcmg NTP]$ ls -ltr total 16 -rw-r--r-- 1 root root 489 Sep 1312:30 99-worker-chrony.bu -rw-r--r-- 1 root root 489 Sep1 3 12:30 99-master-chrony.bu -rw-r--r-- 1 root root 608 Sep 13 12:35 99-worker-chrony.yaml -rw-r--r-- 1 root root 608 Sep 13 12:53 99-master-chrony.yaml

- Once NTP config are applied, the worker and infra nodes will get rebooted and come back online.

[core@drlonvsacmdrcmg NTP]$ ssh -i ~/.ssh/ocp.9 core@drlonvsacmdrcm1.acmdrc.arabbanking.local [core@drlonvsacmdrcmg NTP]$ chronyc sources MS Name/IP address Stratum Poll Reach LastRx Last sample =============================================================================== ^* 10.151.217.1 2 7 377 97 -563us[ -564us] +/- 105ms[

[core@drlonvsacmdrcmg NTP]$ chronyc tracking Reference ID : 0A97D901 (10.151.217.1) Stratum : 3 Ref time (UTC) : Mon Sep 16 10:39:54 2024 System time : 0.000008051 seconds slow of NTP time Last offset : -0.000000964 seconds RMS offset : 0.000097927 seconds Frequency : 13.198 ppm slow Residual freq : -0.000 ppm Skew : 0.095 ppm Root delay : 0.130586922 seconds Root dispersion : 0.020342302 seconds Update interval : 257.9 seconds Leap status : Normal

Repeat the above commands on rest of master nodes to verify the system clock with NTP and make sure "System clock synchronized: yes"

## Configuring an HTPasswd identity provider

- Create a flat file with a user name and hashed password

[core@drlonvsacmdrcm1 ~] mkdir auth_providers [core@drlonvsacmdrcm1 ~] cd auth_providers [core@ drlonvsacmdrcm1 ~] mkdir htpasswd_ auth_providers [core@ drlonvsacmdrcm1 ~] cd htpasswd_ auth_providers [core@ drlonvsacmdrcm1 ~]$ htpasswd -cB users.htpasswd ocpadmin New password: Re-type new password: Adding password for user ocpadmin

The password used for the ocpadmin account hasn’t been disclosed here but the same has been shared with the customer.

- Continue to add or update credentials to the file as required as given in the example below.

[

core@drlonvsacmdrcm1 ~]$ htpasswd -B users.htpasswd developer

New password:

Re-type new password:

Adding password for user developer

- Create an OpenShift Container Platform Secret object that contains the HTPasswd users file

[

core@drlonvsacmdrcm1 htpasswd]$ oc create secret generic htpass-secret --from-file=htpasswd=users.htpasswd -n openshift-config

secret/htpass-secret created

The secret key containing the users file for the --from-file argument must be named

htpasswd

, as shown in the above command.

Create HTPasswd identity provider YAML manifests as below

[

core@drlonvsacmdrcm1 htpasswd

]

$ vi htpasswd-auth-secret.yaml

apiVersion

:

config.openshift.io/v1

kind

:

OAuth

metadata

:

name

:

cluster

spec

:

identityProviders

:

-

name

:

htpasswd_provider

mappingMethod

:

claim

type

:

HTPasswd

htpasswd

:

fileData

:

name

:

htpass-secret

Adding HTPasswd identity provider to the cluster

[core@drlonvsacmdrcm1 htpasswd]$ oc apply -f htpasswd-auth-secret.yaml Warning: oc apply should be used on resource created by either oc create --save-config or oc apply oauth.config.openshift.io/cluster configured

- Access the OCP Web Console and click on the HTPasswd identity provider & login using ocpadmin as the user with the respective password set earlier while creating the user account

- Grant the ocpadmin user account with cluster-admin access as shown below.

[core@drlonvsacmdrcm1 htpasswd]$ oc adm policy add-cluster-role-to-user cluster-admin ocpadmin clusterrole.rbac.authorization.k8s.io/cluster-admin added: "ocpadmin" [core@rlonvsacmdrcm1 htpasswd]$ cd ..

Please store the 'ocpadmin' password in a secured place as it has been granted with cluster-admin privilege to manage the entire OCP cluster.

## Registering the Cluster

- Subscription and support

The cluster will be automatically registered with a 60 day evaluation subscription which does not include support. In order to receive support for your cluster, you will need to register OpenShift Container Platform 4 cluster on cloud.redhat.com.

- To register a disconnected OCP 4 cluster on cloud.redhat.com the cluster profile needs to be created manually as outlined below:

◦ Go to cloud.redhat.com and login with org_admin login credentials.

◦ Go to the Clusters tab and select the three dots near Create cluster and click on the Register cluster button. Alternatively, use https://cloud.redhat.com/openshift/register ◦ ◦ Enter all the required details like cluster-id, number of sockets, memory, etc.

◦ After filling all the required details click on the Register cluster. This will create a cluster profile in disconnected mode.

◦ After that, click on the respective cluster name in the 'Clusters' tab. Edit the Subscription Settings and attach the subscription according to the cluster configuration and environment by scrolling down to 'Subscription settings'.

To get more details, Please visit the below URL:

https://access.redhat.com/solutions/4930131

## Encrypting the ETCD data

- Edit the apiserver object & make an entry under spec as shown below

[

core@drlonvsacmdrcm1]# oc edit apiserver

...

...

spec:

encryption:

type: aescbc

audit:

profile: Default

- Verify if all the resource under openshiftapiserver and kubeapiserver are encrypted as shown below.

[

core@prbahvsacmprodm1]# oc get openshiftapiserver -o=jsonpath='{range

.items[0].status.conditions[?(@.type=="Encrypted")]}{.reason}{"\n"}{.message}{"\n"}'

EncryptionCompleted

All resources encrypted: routes.route.openshift.io, oauthaccesstokens.oauth.openshift.io, oauthauthorizetokens.oauth.openshift.io

[

core@prbahvsacmprodm1]# oc get kubeapiserver -o=jsonpath='{range

.items[0].status.conditions[?(@.type=="Encrypted")]}{.reason}{"\n"}{.message}{"\n"}'

EncryptionCompleted

All resources encrypted: secrets, configmaps

## Backing Up ETCD data

- Login to one of the master node and perform the backup of ETCD data as shown below

[core@master1 ~]$ sudo /usr/local/bin/cluster-backup.sh /home/core/ETCD-Backup-16-09-2024/assets/backup c2d027a039b1fbd493d1689ed36f5b99a37daf764c207e904f5ba3308f2ea879 etcdctl version: 3.4.9 API version: 3.4 found latest kube-apiserver-pod: /etc/kubernetes/static-pod-resources/kube-apiserver-pod-38 found latest kube-controller-manager-pod: /etc/kubernetes/static-pod-resources/kube-controller-manager-pod-10 found latest kube-scheduler-pod: /etc/kubernetes/static-pod-resources/kube-scheduler-pod-9 found latest etcd-pod: /etc/kubernetes/static-pod-resources/etcd-pod-3 {"level":"info","ts":1630482862.6628346,"caller":"snapshot/v3_snapshot.go:119","msg":"created temporary db file","path":"/home/dbpuser/ETCD-Backup-01-05-2024/assets/backup/snapshot_2024-09-16_145106.db.part"}

{"level":"info","ts":"2024-09-16T07:54:22.684Z","caller":"clientv3/maintenance.go:200","msg":"opened snapshot stream; downloading"}

{"level":"info","ts":1630482862.6849284,"caller":"snapshot/v3_snapshot.go:127","msg":"fetching snapshot","endpoint":"https://10.1.108.136:2379"}

{"level":"info","ts":"2024-09-16T07:54:25.207Z","caller":"clientv3/maintenance.go:208","msg":"completed snapshot read; closing"}

{"level":"info","ts":1630482865.365249,"caller":"snapshot/v3_snapshot.go:142","msg":"fetched snapshot","endpoint":"https://10.1.108.136:2379","size":"123 MB","took":2.70228545}

{"level":"info","ts":1630482865.3654559,"caller":"snapshot/v3_snapshot.go:152","msg":"saved","path":"/home/core/assets/backup/snapshot_2024-09-01_075421.db"} Snapshot saved at /home/core/assets/backup/snapshot_2024-09-16_075421.db snapshot db and kube resources are successfully saved to /home/core/assets/backup

- Create a directory to store the ETCD Backup in Bastion Node

[core@master1 ~]$ sudo chown core -R assets/

- Create a directory to store the ETCD Backup in Bastion Node

[core@drlonvsacmdrcm1~]# mkdir ~/ETCD-Backup-16-09-2024/

- Copy the backed up files from the master node to the Bastion node as shown below

[core@bastian]$ scp -r  -i ~/.ssh/ocp4 core@master1.prod.dbp.pnb:~/assets/

[core@drlonvsacmdrcm1 ETCD-Backup]$ ls assets/backup/ snapshot_2024-09-16_145106.db static_kuberesources_2024-09-16_075421.tar.gz

## Deploying and Configuring Openshift Logging

### Install Cluster Logging Operators

[core@drlvsonacmdrcm1]$ mkdir logging [core@drlvsonacmdrcm1]$ cd logging

- Create a Namespace for the Elasticsearch Operator.

[core@drlonvsacmdrcmglogging]$ vi eo-namespace.yaml apiVersion: v1 kind: Namespace metadata: name: openshift-operators-redhat annotations: openshift.io/node-selector: "" labels: openshift.io/cluster-monitoring: "true"

- Creating a namespace for the Cluster Logging operator.

[core@drlonvsacmdrcmglogging]$ vi clo-namespace.yaml apiVersion: v1 kind: Namespace metadata: name: openshift-logging annotations: openshift.io/node-selector: "" labels: openshift.io/cluster-monitoring: "true" [core@drlonvsacmdrcmglogging]$ oc create -f clo-namespace.yaml namespace/openshift-logging created

- Install the Elasticsearch Operator

- In the OpenShift Container Platform web console, click Operators → OperatorHub.

- Choose Elasticsearch Operator from the list of available Operators, and click Install.

- Ensure that the all namespaces on the cluster is selected under Installation Mode.

- Ensure that openshift-operators-redhat is selected under Installed Namespace.

- Select Enable operator recommended cluster monitoring on this namespace.

- Select stable as the Update Channel.

- Select an Approval Strategy to Automatic

- Click Install.

- Verify that the Elasticsearch Operator installed by switching to the Operators → Installed Operators page.

- Ensure that Elasticsearch Operator is listed in all projects with a Status of Succeeded.

- Install the Cluster Logging Operator.

- In the OpenShift Container Platform web console, click Operators → OperatorHub.

- Choose Cluster Logging from the list of available Operators, and click Install.

- Ensure that the A specific namespace on the cluster is selected under Installation Mode.

- Ensure that Operator recommended namespace is openshift-logging under Installed Namespace.

- Select Enable operator recommended cluster monitoring on this namespace.

- Select stable as the Update Channel.

- Select an Approval Strategy to Automatic. Click Install.

- Verify that the Cluster Logging Operator installed by switching to the Operators → Installed Operators page.

- Ensure that Cluster Logging is listed in the openshift-logging project with a Status of Succeeded.

### Create Cluster Logging Instance

- Create a cluster logging instance YAML manifest

- Create a Cluster Logging instance

[core@drlonvsacmdrcmglogging]$ vi clo-instance.yaml apiVersion: "logging.openshift.io/v1" kind: "ClusterLogging" metadata: name: "instance" namespace: "openshift-logging" spec: collection: logs: fluentd: resources: limits: cpu: 500m memory: 1Gi requests: cpu: 500m memory: 1Gi type: fluentd curation: curator: nodeSelector: node-role.kubernetes.io/infra: "" resources: limits: memory: 1Gi requests: cpu: 200m memory: 200Mi schedule: 30 3 * * * type: curator logStore: elasticsearch: nodeCount: 3 nodeSelector: node-role.kubernetes.io/infra: "" redundancyPolicy: SingleRedundancy resources: limits: cpu: 2000m memory: 16Gi requests: cpu: 1500m memory: 16Gi storage: size: 300Gi storageClassName: nfs01 retentionPolicy: application: maxAge: 3d audit: maxAge: 3d infra: maxAge: 3d type: elasticsearch managementState: Managed visualization: kibana: nodeSelector: node-role.kubernetes.io/infra: "" replicas: 1 resources: limits: memory: 1Gi requests: cpu: 500m memory: 1Gi type: kibana

- Create the ClusterLogging instance object as shown below

[core@drlonvsacmdrcmglogging]$ oc create -f clo-instance.yaml clusterlogging.logging.openshift.io/instance created

- Verify the cluster logging pods deployed in the openshift-logging project

[core@drlonvsacmdrcmg ~]$ oc get po -o wide -n openshift-logging NAME READY STATUS RESTARTS AGE IP NODE NOMINATED NODE READINESS GATES cluster-logging-operator-64654fbd86-7s454 1/1 Running 0 177m 10.129.2.7 drlonvsacmdrcw1.acmdrc.arabbanking.local <none> <none> collector-6hpjm 1/1 Running 2 4d21h 10.128.1.151 drlonvsacmdrcm1.acmdrc.arabbanking.local <none> <none> collector-cc898 1/1 Running 2 4d21h 10.131.0.37 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> collector-m565l 1/1 Running 2 4d21h 10.129.2.68 drlonvsacmdrcw1.acmdrc.arabbanking.local <none> <none> collector-n2h29 1/1 Running 2 4d21h 10.130.1.160 drlonvsacmdrcm3.acmdrc.arabbanking.local <none> <none> collector-n6prl 1/1 Running 2 4d21h 10.130.2.137 drlonvsacmdrcw2.acmdrc.arabbanking.local <none> <none> collector-xbrkq 1/1 Running 2 4d21h 10.129.0.97 drlonvsacmdrcm2.acmdrc.arabbanking.local <none> <none> collector-xp9cf 1/1 Running 2 4d21h 10.128.2.248 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> elasticsearch-cdm-j6bbys15-1-f47d4cc5b-s7v9x 2/2 Running 0 177m 10.128.2.19 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> elasticsearch-cdm-j6bbys15-2-b75f6d77-b2fjz 2/2 Running 0 177m 10.128.2.16 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> elasticsearch-cdm-j6bbys15-3-754df85bdb-v8nbc 2/2 Running 0 177m 10.128.2.15 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> elasticsearch-im-app-28774710-rpn2d 0/1 Completed 0 13m 10.131.0.65 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> elasticsearch-im-audit-28774710-nxgp5 0/1 Completed 0 13m 10.131.0.64 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> elasticsearch-im-infra-28774710-pl5wd 0/1 Completed 0 13m 10.131.0.63 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> kibana-86dc6b855c-mm7vf 2/2 Running 0 177m 10.128.2.18 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none>

- Verify PVCs created for Elasticsearch components

[core@drlonvsacmdrcmg ~]$ oc get pvc -n openshift-logging NAME STATUS VOLUME CAPACITY ACCESS MODES STORAGECLASS AGE elasticsearch-elasticsearch-cdm-j6bbys15-1 Bound acmdrcelk1 300Gi RWO nfs01 5d3h elasticsearch-elasticsearch-cdm-j6bbys15-2 Bound acmdrcelk2 300Gi RWO nfs01 5d3h elasticsearch-elasticsearch-cdm-j6bbys15-3 Bound acmdrcelk3 300Gi RWO nfs01 5d3h

### Viewing cluster logs from the Kibana dashboard

- To define index patterns and create visualizations in Kibana

- In the OpenShift Container Platform console, click the Application Launcher and select Logging.

- Create your Kibana index patterns by clicking Management → Index Patterns → Create index pattern.

  - Users must manually create index patterns to see logs for their projects. Users should create a new index pattern named app and use the @timestamp time field to view their container logs.

  - Admin users must create index patterns for the app, infra, and audit indices using the @timestamp time field.

- Create Kibana Visualizations from the new index patterns.

- To view logs in Kibana

- In the OpenShift Container Platform console, click the Application Launcher and select
Logging.

- Log in using the same credentials you use to log in to the OpenShift Container Platform console.

- In Kibana, click Discover.

- Select the index pattern you created from the drop-down menu in the top-left corner: app, audit, or infra. The log data displays as time-stamped documents.

- Expand one of the time-stamped documents.

## Applying a custom Alertmanager configuration with the SMTP details

• Print the currently active Alertmanager configuration into a file alertmanager.yaml

[core@drlvsonacmdrcm1]$ mkdir SMTP [core@drlvsonacmdrcm1 SMTP]$ cd SMTP [core@drlvsonacmdrcm1 SMTP]$ oc -n openshift-monitoring get secret alertmanager-main --template='{{ index .data "alertmanager.yaml" }}' ¦ base64 -d > alertmanager.yaml

• Edit the configuration in the exported alertmanager.yaml file

"global": "resolve_timeout": "5m" "smtp_from": "CMS.Monitoring@bank-abc.com" "smtp_smarthost": "10.150.163.236:25" "smtp_require_tls": false "inhibit_rules": - "equal": - "namespace" - "alertname" "source_match": "severity": "critical" "target_match_re": "severity": "warning" - "equal": - "namespace" - "alertname" "source_match": "severity": "warning" "receivers": - "name": "Default" "email_configs": - "to": "GPH.Monitoring@bank-abc.com" "headers": "subject": 'CMS-DRC Alert {{ template "email.default.subject" . }}' "route": "group_by": - "namespace" "group_interval": "5m" "group_wait": "30s" "receiver": "Default" "repeat_interval": "8h" "routes": - "match": "alertname": "Watchdog" "receiver": "Default" - "match": "severity": "critical" "receiver": "Default”

Apply the new edited configuration as shown below.

$ oc -n openshift-monitoring create secret generic alertmanager-main --from-file=alertmanager.yaml --dry-run -o=yaml ¦ oc -n openshift-monitoring replace secret --filename=- secret/alertmanager-main replaced

## Removing the kubeadmin user

After you define an identity provider and create a new cluster-admin user, you can remove the kubeadmin to improve cluster security.

If you follow this procedure before another user is a cluster-admin, then OpenShift

Container Platform must be reinstalled. It is not possible to undo this command.

Prerequisites

- You must have configured at least one identity provider.

- You must have added the cluster-admin role to a user.

- You must be logged in as an administrator.

- Retrieve the kubeadmin secret details as shown below

[core@basian-node ~]$ oc get secrets kubeadmin -n kube-system NAME TYPE DATA AGE kubeadmin Opaque 1 97d

Remove the kubeadmin secret as shown below

[core@drlvsonacmdrcm1e ~]$ oc delete secrets kubeadmin -n kube-system secret "kubeadmin" deleted

Verify if the kubeadmin secret no more exists as shown below

[core@drlvsonacmdrcm1e ~]$ oc get secrets kubeadmin -n kube-system Error from server (NotFound): secrets "kubeadmin" not found

## Moving monitoring components from Worker to Infra labeled nodes

Create a configmap as shown below

[core@drlvsonacmdrcm1e ~]$vi cluster-monitoring-config.yaml apiVersion: v1 kind: ConfigMap metadata: name: cluster-monitoring-config namespace: openshift-monitoring data: config.yaml: ¦+ alertmanagerMain: volumeClaimTemplate: metadata: name: alertmanager spec: storageClassName: nfs01 volumeMode: Filesystem resources: requests: storage: 5Gi nodeSelector: node-role.kubernetes.io/infra: "" prometheusK8s: volumeClaimTemplate: metadata: name: prometheus spec: storageClassName: nfs01 volumeMode: Filesystem resources: requests: storage: 300Gi retention: 30d nodeSelector: node-role.kubernetes.io/infra: "" prometheusOperator: nodeSelector: node-role.kubernetes.io/infra: "" grafana: nodeSelector: node-role.kubernetes.io/infra: "" k8sPrometheusAdapter: nodeSelector: node-role.kubernetes.io/infra: "" kubeStateMetrics: nodeSelector: node-role.kubernetes.io/infra: "" telemeterClient: nodeSelector: node-role.kubernetes.io/infra: "" openshiftStateMetrics: nodeSelector: node-role.kubernetes.io/infra: "" thanosQuerier: nodeSelector: node-role.kubernetes.io/infra: ""

[core@drlvsonacmdrcm1e ~]$ oc create -f cluster-monitoring-config.yaml

Verify if all the core monitoring components have been moved to the dedicated nodes (infra) as shown below

[core@drlonvsacmdrcmg ~]$ oc get po -o wide NAME READY STATUS RESTARTS AGE IP NODE NOMINATED NODE READINESS GATES alertmanager-main-0 6/6 Running 0 3h 10.131.0.10 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> alertmanager-main-1 6/6 Running 0 3h2m 10.128.2.8 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> cluster-monitoring-operator-f5974dd98-2glfr 1/1 Running 0 84m 10.128.0.32 drlonvsacmdrcm1.acmdrc.arabbanking.local <none> <none> kube-state-metrics-54bbbb5488-t82mx 3/3 Running 0 3h 10.128.2.14 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> monitoring-plugin-6c5cfd47bf-dtgwz 1/1 Running 0 3h 10.129.2.8 drlonvsacmdrcw1.acmdrc.arabbanking.local <none> <none> monitoring-plugin-6c5cfd47bf-pk855 1/1 Running 0 3h 10.128.2.20 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> node-exporter-2x2ck 2/2 Running 6 6d21h 10.151.24.84 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> node-exporter-4zntd 2/2 Running 6 6d21h 10.151.24.79 drlonvsacmdrcm2.acmdrc.arabbanking.local <none> <none> node-exporter-6bfgc 2/2 Running 6 6d21h 10.151.24.83 drlonvsacmdrcw2.acmdrc.arabbanking.local <none> <none> node-exporter-b4tdc 2/2 Running 6 6d21h 10.151.24.82 drlonvsacmdrcw1.acmdrc.arabbanking.local <none> <none> node-exporter-h2wbh 2/2 Running 6 6d21h 10.151.24.81 drlonvsacmdrcm3.acmdrc.arabbanking.local <none> <none> node-exporter-hrzwf 2/2 Running 4 6d3h 10.151.24.85 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> node-exporter-j7q54 2/2 Running 6 6d21h 10.151.24.78 drlonvsacmdrcm1.acmdrc.arabbanking.local <none> <none> openshift-state-metrics-649d97fd7b-xw9vz 3/3 Running 0 3h 10.128.2.12 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> prometheus-adapter-85696d9dfb-4jjgf 1/1 Running 0 3h 10.131.0.7 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> prometheus-adapter-85696d9dfb-nzrwg 1/1 Running 0 3h2m 10.128.2.11 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> prometheus-k8s-0 6/6 Running 0 3h2m 10.128.2.6 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> prometheus-k8s-1 6/6 Running 0 3h 10.131.0.9 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> prometheus-operator-84494bf4c9-2fvr7 2/2 Running 0 3h 10.128.2.13 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> prometheus-operator-admission-webhook-5cf6d4f94d-b27zq 1/1 Running 0 3h2m 10.128.2.5 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> prometheus-operator-admission-webhook-5cf6d4f94d-ll22g 1/1 Running 0 3h 10.131.0.11 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> telemeter-client-6877bfb864-qrxvb 3/3 Running 0 3h 10.128.2.17 drlonvsacmdrcw3.acmdrc.arabbanking.local <none> <none> thanos-querier-7f844b45bf-f9lnz 6/6 Running 0 3h 10.131.0.8 drlonvsacmdrcw4.acmdrc.arabbanking.local <none> <none> thanos-querier-7f844b45bf-lg7br 6/6 Running 0 3h2m 10.128.2.10 drlonvsacmdrcw3.acmdrc.arabbanking.local <none>

## Configuring the default scheduler to control pod placement

Edit the scheduler configuration & add the below stated line under spec section and save the configuration.

[core@drlvsonacmdrcm1e ~]$ oc edit scheduler cluster ... spec: defaultNodeSelector: node-role.kubernetes.io/worker= ...

## Etcd backup Cronjob

Create Backup Cronjob using below yaml and apply the yaml.

kind: CronJob apiVersion: batch/v1 metadata: name: openshift-backup namespace: ocp-etcd-backup labels: app: openshift-backup spec: schedule: "* * * * *" concurrencyPolicy: Forbid successfulJobsHistoryLimit: 5 failedJobsHistoryLimit: 5 jobTemplate: metadata: labels: app: openshift-backup spec: backoffLimit: 0 template: metadata: labels: app: openshift-backup spec: containers: - name: backup image: "registry.redhat.io/openshift4/ose-cli" command: - "/bin/bash" - "-c" - oc get no -l node-role.kubernetes.io/master --no-headers -o name ¦ head -n 1 ¦xargs -I {} -- oc debug {} --to-namespace=ocp-etcd-backup -- bash -c 'chroot /host rm -rf /home/core/backup && chroot /host mkdir /home/core/backup && chroot /host sudo -E mount -t nfs <nfs-server-IP>:<shared-path> /home/core/backup && chroot /host sudo -E /usr/local/bin/cluster-backup.sh /home/core/backup && chroot /host sudo -E find /home/core/backup/ -type f -mmin +"1" -delete' restartPolicy: "Never" terminationGracePeriodSeconds: 30 activeDeadlineSeconds: 500 dnsPolicy: "ClusterFirst" serviceAccountName: "openshift-backup" serviceAccount: "openshift-backup"

After creating CronJob, you can force the execution for validation with the command:

oc create job bckup --from=cronjob/openshift-backup

Thank You

- Visit us at www.integramicro.com